FLAT FLOOR & FULL WALL SLIDE-OUTS

2 Types of Flat Floor Slide-Outs

- Full Wall – uses two motors with brakes and encoders, four cog wheels, 4 actuating arms, and a KIB controller

- Basic – Uses one motor with a brake, two cog wheels, two actuating arms, and a C-Storm controller

All Flat Floor Slide-Outs

- Uses Rack and Pinion System

- Ignition must be turned OFF  and Parking Brake SET to operate

- Uses a ramp to lower slide-out to same level as the main floor

Retracting the Slide-Out Manually

- Manual Retraction: Method #1 – Loosen all Trantorque bushings and push slide-out in by hand, Re-torque Bushings to 175 ft lbs

- Manual Retraction: Method # 2  - Remove the rubber boot and brake from the motor(s), use a screw gun with a 5/8” socket to turn gear reduction nut. (Alternate between motors or have a helper and 2nd screw gun and operate both motors on Full Wall)

Squaring a Flat Floor (NON- Full Wall Slide-Out)

- If a Trantorque bushing slips it will cause the room to bind or extend/retract unequally

- If a Trantorque bushing slips on a Full Wall slide out, do this, then set the “Soft Limits”

- Extend the slide-out to just before full extension and measure from the “Z” trim to the slide-out trim.

- Loosen the Trantorque bushing(s) and manually position the slide-out so that both ends are equal.

- Torque the Trantorque bushing to 175 ft lbs

- Using a marker, draw a line across the bushing and shaft

- Run the slide-out several times and verify the bushing is not slipping on the shaft

Adjusting the Reveal Front to Rear

- Over time the slide-out may drift forward or backward out of center

- This adjustment affects the slide-out in the “In” or “Retracted Position”

- Check the reveal of the slide-out trim in relation to the “Z” trim

- Fully extend the slide-out

- Use a grease pencil to mark your starting point on the activating tubes

- Loosen the nut in the center of the activating tubes

- Move the tube in the opposite direction you want the slide-out to move

- Move all tubes the same amount and re-tighten the nuts after each step

- Extend and retract the slide-out several times to ensure the adjustments are correct

Adjusting the Reveal Up and Down

- Over time the Slide-out may settle and become too high or too low

- This adjustment affects the slide-out in the “In” or “Retracted Position”

- Watch paint lines and reveal before, during and after adjustment

- Loosen the jam nuts on the outer bolts Loosen the center bolt of the activating tube

- Tighten the outer bolts to raise the slide-out

- Loosen the outer bolts to lower the slide-out

- Re-tighten jam nuts and center bolt

- Run slide-out several times to ensure correct adjustment

Adjusting the “Saddle” and “Stop Rod”

- Needed if the slide-out is not sealing to the interior wall

- The “Saddle” adjustment affects the top of the slide-out

- The “Stop Rod” adjustment affects the bottom of the slide-out

- This adjustment affects the slide-out in the “out” or “Extended” position

- This is accessed through the baggage compartment directly under each actuating arm

- Extend the slide-out half way

- To tilt the top of the slide-out closer to the interior wall adjust the “Saddle” bolt up

- To tilt the top of the slide-out farther away from the interior wall adjust the “Saddle” bolt down

- To adjust the bottom of the slide-out adjust the “Stop Rod” in or out accordingly

Replacing the Motor

- Level the coach on the jacks

- Jack up the slide-out just enough to take the weight off the cog wheel and shaft

- Loosen the Trantorque bushing and set screws on the cog wheel, and bearing retainer

- Slide the cog wheel through the shaft to allow shaft to be disassembled

- Loosen the 4 bolts and slide the motor off the shaft

- Reverse the procedure to reassemble

- Be sure to use “Blue” Loctite on the motor bolts and torque to 33 ft lbs

- Ensure the shaft is clean, and torque the Trantorque bushing to 175 ft lbs

Replacing the Rollers

- DO NOT use this procedure on a Full Wall Slide out. (It is too heavy to use these blocks)

- Extend the slide-out all the way

- Place blocks on the roof of the slide-out

- Retract the slide-out, making sure the blocks are not gouging the roof and contacting the “Z” trim correctly. This will tilt the slide-out and allow for access to the rollers

- Install a block under the slide-out for safety and replace the roller

- The same concept applies for Full Wall slide-outs. With the slide-out approximately ½ way in and the help of 4 or 5 people, manually tilt the slide-out up, by pushing on the top of the slide-out, from inside the coach. Then place blocks under it, to hold it up while removing the rollers.